# SECTION 08 34 00 – SPECIALTY DOORS

## PART 1 – GENERAL

### 1.01 SECTION INCLUDES

A. Fabric roll-up doors.
B. Screen roll-up doors.

### 1.02 RELATED SECTIONS

A. None.

### 1.03 REFERENCES

A. None.

### 1.04 SUBMITTALS

A. Product Data: Submit manufacturer’s product literature for doors, including performance characteristics and installation requirements.
B. Shop Drawings: Indicate elevations, details, dimensions, and installation requirements.
C. Samples: Submit fabric and finish samples for selection.

### 1.05 QUALITY ASSURANCE

A. Manufacturer Qualifications: Minimum three (3) years producing roll-up doors of similar type.
B. Installer Qualifications: Minimum two (2) years installing comparable roll-up door systems.

### 1.06 WARRANTY

A. Provide manufacturer’s standard warranty.
B. Provide minimum three (3) year warranty covering manufacturer’s defects.

## PART 2 – PRODUCTS

### 2.01 MANUFACTURER

A. Basis of Design: Screen-Pro™ or Vinyl-Pro™ by TMI, a Senneca brand. 9229 Shaver Rd, Portage, MI 49024 Phone: 800-543-4455

B. Substitutions: None without prior approval

2.02 PERFORMANCE REQUIREMENTS

A. Operation: Manual or motorized activation, per size and use.
B. Maximum Sizes:
   1. Motorized: up to 264" x 264".
   2. Chain hoist: up to 168" x 168".
   3. Spring-loaded: up to 144" x 144".

2.03 MATERIALS
A. Curtain Material: 11 oz. PVC-coated vinyl mesh (standard) or solid fabric
   1. Options: Bird mesh, heavier vinyl (18.5–40 oz.), or clear PVC window panels.
   2. Modular panel system for seasonal replacement.

B. Side Columns: Extruded aluminum guide tracks, 4-1/2" deep, with weatherseal and UHMW insert.

C. Bottom Bar: Extruded aluminum breakaway type, resettable after impact.

D. Mounting Options:
   1. In-Jamb.
   2. Face-of-Wall.
   3. Inside Stand-Off (around existing doors).

E. Operators and Controls:
   1. Manual: Spring-loaded tube or chain hoist.
   2. Motorized: In-tube or external duty motor, UL 325 compliant.
   3. Controls: Rocker switch standard; optional remote, motion sensors, timers.

F. Safety Devices (optional for motorized): Photo-eye, reversing safety edge.

PART 3 – EXECUTION

3.01 EXAMINATION

A. Verify opening dimensions and substrate conditions.

B. Do not proceed until unsatisfactory conditions are corrected.

3.02 PREPARATION

A. Ensure jambs are plumb and square.

B. Provide structural support as required for anchorage.

3.03 INSTALLATION

A. Install doors per manufacturer’s instructions.

B. Reinforce hollow metal jambs and sheet metal walls at hardware locations.

C. Adjust doors for smooth operation.

3.04 FIELD QUALITY CONTROL

A. Test door operation in presence of Owner’s representative.

B. Demonstrate safety features and manual override.

3.05 CLEANING AND PROTECTION

A. Clean exposed surfaces after installation.

B. Protect units from damage until acceptance.

END OF SECTION 08 34 00